## How to download/upgrade firmware to ESD tester

- Plug download cable to tester, then join DB9 female to RS232 cable. After that, plug RS232 USB to computer

- In your PC, extract PL2303_Prolific_DriverInstaller_v1.5.0.zip file. Then double click PL2303_Prolific_DriverInstaller_v1.5.0.exe to install driver. If your PC is Windows 10, you may skip this step because driver may install automatically

- Search device manager and open it

- Expand Ports drop down, check is it show Prolific USB-to-Serial Comm Port in list. If not found then unplug RS232 and replug RS232 cable again

After install driver and plug RS232 cable to PC should found Prolific USB-to-Serial Comm Port

- Extract tinybld198.zip file, double click tinybldWin.exe to open program

- Press Browse button then select rcFSTJ18F46201Gig100KV40.52.7.hex file and press Open button

- Press CheckPIC button to check is it can read IC without problem. If program can read to tester without problem then it should found IC type of tester otherwise it will show error. If program can success found IC type and IC type behind is 4620 then can process to next step otherwise please stop here and don’t continue.

- If can read IC without problem then press Write Flash button to start download firmware to ESD tester and wait status bar run finish

- Once firmware download finish ESD tester will reset itself

- After ESD tester reset complete then try to test